The supermode:

in this (One PCB Version) you have a conplete Bandido2 umax, with any expansions:
- Batterie Test (you dont need, no set switch to Batt-Test)
- extern On/Off switch (you dont need this, set a bridge to jumper on-off)
- Aux Freq (Orginal 625Hz with bridge on Pin2 and Pin3 on Jumper Aux-Freq, with a bridge on Pin2 and Pin1 you have 312,5Hz AuxFrequenz)
- a Jack

you can this one pcb Version upgrade to the "supermode":
change the C10, C13, C14, C16 und C18 to:
C10 - 22pF
C13 - 10pF
C14 - 8,2pF
C16 - 39pF
C18 - 22pF

alternative Supermode (not testet):
C10 - 4,7pF
C13 - 4,7pF
C14 - 4,7pF
C16 - 10pF
C18 - 4,7pF

or you dont build the C10, C13, C14, C16 und C18 in the PCB Board and build yourself an little extern board with cable from the connections: C10, C13, C14, C16 und C18 and then with a switch to change beetween Orginal-Supermode-SupermodeAlternative.


With this One-Version PCB Board you can build the Orginal Bandido2 umax or the Orginal BandidoII umax with BatterieTest/anotherAux/.. or you can it modding to Supermode - its your choise